FORKLIFT SAFETY PROGRAM (FSP)

HANDS ON EXAMINATION

Rate as:    Pass (P), Needs Improvement (I), or Not Applicable (N/A)

- Ability to perform Operator’s Daily Checklist					_____

- Wears Safety Equipment/Seat Belt (if applicable)					_____

- Proper use of driving controls

A.    Steering										_____

B.    Speed control									_____

C.    Brakes/Deadman Pedal								_____

D. Plugging	 (if applicable)								_____

- Manuevering unloaded

A.    Smooth starting and stopping							_____

B.    Smooth steering and turning (no sharp turns / slowing for turns)			_____

C.    Proper speed									_____

D.    Proper use of hands and body positioning (both directions)			_____

E.    Looks in the direction of travel							_____

F.    Watches for rear end swing around corners					_____

G.    Slows, sounds horn and yields right of way to pedestrians			_____

- Picking up a pallet

A.    Checks to make sure load is secure						_____

B.    Spreads forks as far apart as possible						_____

C.    Lines up forks properly								_____

D.    Moves truck forward so shank butts up to pallet 					_____

E.    Lifts load properly (correct height) and tilts forks so load is level		_____

F.    Looks in direction of travel and slowly moves away from rack			_____

- Manuevering loaded

A.    Carries load as low as possible							_____

B.    Smooth steering and turning (no sharp turns / slowing for turns)			_____

C.    Proper speed									_____

D.    If vision is obstructed by tall load, travels with load trailing			_____

E.    Looks in the direction of travel							_____

F.    Watches for swing of rear end and load						_____

G.    Slows, sounds horn and yields right of way to pedestrians			_____

- Depositing pallet in lowest rack (or staging)

A.    Lines forks up with pallet slot							_____

B.    Raises the forks so pallet is just above rack beam (or ground surface)		_____

C.    Slowly moves truck forward until pallet is in line				_____

D.    Lowers forks until pallet touches the rack (or ground surface)			_____

E.    If necessary, tilts the forks down until pallet rests on rack/ground		_____

F.    Lowers forks until forks are free of pallet						_____

G.    Looks in direction of travel amd slowly moves away from rack/pallet		_____

H.    Lowers the forks as low as possible before traveling				_____

- Picking up pallet from high rack

A.    Checks that load weight is within vehicle capacity				_____

B.    Spreads forks as far apart as possible						_____

C.    Lines forks up with pallet opening of lowest pallet in rack			_____

D.    Looks above for overhead obstructions						_____

E.    Raises the forks until they are in line with pallet opening				_____

F.    Slowly moves truck forward until pallet is in line					_____

G.    Lifts load slowly several inches							_____

H.    Tilts the forks so that the load is level						_____

I.    Looks in direction of travel amd slowly moves away from rack			_____

J.    Lowers the forks as low as possible before traveling				_____

- Depositing pallet in a high rack

A.    Lines forks up with pallet opening of lowest pallet in rack			_____

B.    Looks above for overhead obstructions						_____

C.    Raises the forks so that the pallet is just above the rack beam			_____

D.    Slowly moves truck forward or extends fork carriage until pallet is in line	_____

E.    Lowers the forks until the pallet touches the rack					_____

F.    If necessary, tilts the forks down until the pallet rests on the rack		_____

G.    Lowers the forks until the forks are free of the pallet				_____

H.    Looks in direction of travel amd slowly moves away from rack			_____

I.    Lowers the forks as low as possible before traveling				_____