FORKLIFT SAFETY PROGRAM (FSP)

NARROW AISLE (537 Swing Reach) WRITTEN EXAMINATION

I. Multiple Choice Questions Section Instructions-- Read each question CAREFULLY and then circle the letter of the MOST CORRECT answer from the choices below.

- When stopping a truck in an emergency, the operator should first:

- A.  Move the directional/speed control through neutral to opposite direction (plugging)

- B.  Quickly turn the ignition key to off.

- C   Release the travel deadman pedal.

- D.  Hit the quick release control that connects the battery to the truck.

- What should a driver do before driving through a blind intersection or at cross aisles ?

- A.  Quickly drive through the intersection to keep out of anyone else’s way..

- B.  Sound the horn, move slowly.

- C.  Stop, sound the horn, move slowly.

- D.  Move slowly.

- If your loaded truck was traveling in fork direction, then stopped suddenly, what effect would momentum have on the load ?

- A.  It would cause it to shift to the right.

- B.  It would cause it to shift to the left.

- C.  It would cause it to keep moving in the same direction as travel (fork direction).

- D.  It would cause it to move in the opposite direction from travel (tractor direction).

- What should you do if you find any problems with the vehicle during the Daily Inspection Checklist ?

- A. Tell the next driver.

- B.  Leave a note for your supervisor, then use the truck until you hear from him/her.

- C.  Report the problem to your supervisor, then do not operate the truck.

- D.  Find another truck to drive so that you do not have to deal with the problem.

- You must wear the safety harness:

- A.  Only when you are going to raise the operator platform.

- B.  When you are carrying other passengers.

- C.  When you get scared.

- D.  At all times while operating this type of equipment.

- II. TRUE OR FALSE (Circle T or F whichever is more correct)

- If you do not complete initializing procedures after turning on this equipment,

- your speed of travel will be limited. 	T  F

- When you face forward toward the fork carriage, the hand control at your right operates the

- direction & speed control, the main lift & lower system, the horn and auto manual

- rocker switch. 	T  F

- It’s okay not to look behind you when backing up just a few feet.    		T  F

- Since the rear wheel guides the turning of the truck, the rear end of the truck will

tend to swing wide in a turn.		T  F

- Plugging is the preferred method of changing direction (as well as stopping).		T  F

- To rotate the fork carriage from right to left, rotate the thumb control counter clockwise

- on the load handler control while pulling the load handler control back. 		T  F

- You can drive at full speed with a load slightly elevated as long as you can see over

- the load. 		T  F

- The horn is a 360 degree warning device to others nearby.		T  F

- The narrow aisle has two deadman pedals, both of which have to be depressed in

- order for the truck to move forward or in reverse.		T  F

- The five parts the visual inspection are:  tires, battery, lift/lower system, controls,

- general safety.		T  F

- The six parts of the operational inspection are:  horn, steering, directional/speed

- control, lift/lower system, brakes, general safety.		T  F

- I must always wear a safety hard hat and a safety harness while operating this

- equipment.		T  F

- I will follow guidance given to me from all supervisors, forklift trainers, security

personnel and the Safety Officer.		T  F

- The load should not be handled if it is so unstable or so insecure that it might come

- loose while being moved.		T  F

- When raising the operator platform to place or retrieve a load, the operator must

look above to make sure that the operator platform and mast do not hit any overhead

obstructions.  		T  F