FORKLIFT SAFETY PROGRAM (FSP)

REACH TRUCK/ORDER PICKER WRITTEN EXAMINATION

I. Multiple Choice Questions Section Instructions-- Read each question CAREFULLY and then circle the letter of the MOST CORRECT answer from the choices below.

- What are the five parts of the visual inspection of the Operator’s Daily Checklist?

- A.  paint job, seat covers, brakes, horn, load center

- B.  tires, battery, lift/lower system, controls, general safety

- C.  controls, center of gravity, brakes, load limit, steering

- D.  horn, steering, directional/speed control, lift/lower system, brakes

- What are the six parts of the operational inspection of the Operator’s Daily Checklist ?

- A.  paint job, seat covers, brakes, horn, load center, load limit

- B.  tires, battery, lift/lower system, controls, general safety, hour meter

- C.  controls, center of gravity, brakes, load limit, steering, battery

- D. horn, steering, directional/speed control, lift/lower system, brakes, general safety

- What is the best way to minimize the effects of centrifugal force in a turn ?

- A.  Accelerate as quickly as possible.

- B.  Speed up when apporaching the turn.

- C.  Make the turn as sharply as possible.

- D.  Make the turn slowly.

- What should you do if your supervisor walks under an elevated load ?

- A.  Nothing, because your supervisor should know better.

- B.  Begin to lower the load.

- C. Get off the truck, walk over to your supervisor, and politely ask him or her to move.

- D.  Stop moving and ask your supervisor to move immediately.

- What is the best way to minimize the effects of momentum when starting and stopping a truck ?

- A.  Accelerate as quickly as possible.

- B.  Face away from the load.

- C.  Lean towards the direction of travel.

- D.  Avoid sudden starts and stops.

- II. TRUE OR FALSE (Circle T or F whichever is more correct)

- When stopping in an emergency, the operator should first release the deadman pedal.	T  F

- When you complete the Operator’s Daily Checklist, part of the battery check is ensuring that the battery is fully charged.  If it isn’t, the battery should be recharged immediately.	T  F

- When a load is elevated, the combined center of gravity will move down. 	T  F

- The horn is a 360 degree warning device to others nearby.  	  	T  F

- The load capacity is the maximum weight that can be carried safely at a designated load center and elevated height.		T  F

- Before raising the mast, you should check overhead for sprinkler heads, beams or other obstructions.		T  F

- If you have just moved forward to pick up a load and are sure that the path is clear, you don’t have to look behind you when you back up.		T  F

- If you took a sharp left turn, centrifugal force will cause the center of gravity to shift to the right momentarily.		T  F

- Always yield to pedestrians.		T  F

- If you operate an order picker, you must wear a safety harness.		T  F

- It is okay to drive up to other employees standing next to the racking if they are there to help you load your truck.	 	T  F

- Forks should be placed under the load as far as possible, and if the truck is equipped with mast tilt or fork tilt, the load should be slightly tilted.		 T  F

- You must carry a load as low as possible to increase stability; however, if you are traveling without a load, you do not need to worry about the level of your forks. 		T  F

- A truck may lose its balance when a load is too heavy or when the load is carried too far from the shank of the forks.		T  F

- When you cannot safely see over a load, you should turn around, face away from the load and drive in that direction.		T  F