FORKLIFT/INDUSTRIAL TRUCK SAFETY CHECKLIST

This safety checklist will help employees and supervisors follow minimal safety practices. This list is not meant to be comprehensive nor is it meant to form part of any official self-assessment practice. Where appropriate, local safety offices and supervisors are encouraged to add to these checklists. Relevant references are noted after each question. These regulations apply only to installations or equipment used on temporary and permanent job sites.

# General                                                 			OK      Action Needed

Do industrial trucks acquired after Feb. 15, 1972 meet

the design requirements in "American National

Standard for Powered Industrial Trucks, Part II,

ANSI B56.1-1969?"                                      			----     -----------

29 CFR 1910.178 (a) (2)

Has the manufacturer provided written approval for

modifications that affect the capacity and safety

operations of the equipment?                          		-----     -----------

29 CFR 1910.178 (a) ( 4)

Do industrial trucks have labels designating

approval for use in various hazardous and/or

nonhazardous locations?                                		-----     -----------

29 CFR 1910.178 (a) (3) and (7)

Designations

Are supervisors and procurers of equipment aware

of the eleven designations of industrial trucks

or tractors (D, DS, DY, E, ES, EE, EX, G, GS, LP,

and LS)?                                               			-----     -----------

29 CFR 1910.178 (b)

# Designated Use of Requirements

Are supervisors and operators knowledgeable about

the use of industrial trucks in various locations?     		-----     -----------

29 CFR 1910.178 (c) (1)

# Fuel Handling and Storage Requirements

Is the storage and handling of liquid fuels in

accordance with NFPA Flammable and Combustible

Liquids Code (NFPA No. 58-1969)?                       		-----     -----------

29 CFR 1910.178 (f) (1)

Is the storage and handling of liquefied

petroleum gas fuel in accordance with NFPA Storage

and Handling of Liquefied Petroleum Gases (NFPA No.

58-1969)?                                              			----     -----------

29 CFR 1910.178 (f) (2)

# Changing and Charging Storage Batteries

Are battery-charging installations located in

areas designated for that purpose?                     		-----     -----------

29 CFR 1910.178( g) (1)

Are facilities provided for flushing and

neutralizing spilled electrolyte?                      		-----     -----------

29 CFR 1910.178 (g) (2)

Are facilities provided for adequate ventilation for

dispersal of fumes from gassing batteries?             		-----     -----------

29 CFR 1910.178 (g) (2)

Is proper handling equipment (conveyor and hoists)

provided for handling batteries?                       		-----     -----------

29 CFR 1910.178 (g) (4)

Is a carbon filter or siphon provided for handling

electrolyte?                                           			-----     -----------

29 CFR 1910.178 (g) (6)

Is care taken to ensure that vent caps are

functioning when charging batteries?                   		-----     -----------

Note: The battery (or compartment) cover(s)

shall be open to dissipate heat.

29 CFR 1910.178 (g) (9)

Is smoking prohibited in the charging area?            		-----     -----------

29 CFR 1910.178 (g) (10)

Are precautions taken to prevent open flames,

sparks, or electric arcs in battery-charging areas?    	----     -----------

29 CFR 1910.178 (g) (11)

Are tools and other metallic objects kept away from

the tops of uncovered batteries?                       		-----     -----------

29 CFR 1910.178 (g) (12)

Dockboards (bridge plates)

Are portable and powered dockboards strong enough

to carry the load imposed on them?                     		-----     -----------

29 CFR 1910.30 (a) (i)

Are portable dockboards secured in position, either

by being anchored or equipped with devices that

will prevent slippage?                                 			-----     -----------

29 CFR 1910.30 (a) (2)

Are handholds or other effective means provided

on portable dockboards to ensure safe handling?        	-----     -----------

29 CFR 1910.30 (a) (4)

Is positive protection provided to prevent

railroad cars from being moved while dockboards

or bridge plates are in position?                      		-----     -----------

29 CFR 1910.30 (a) (5)

# Trucks and Railroad Cars

Do trucks have positive protection to prevent them

from moving during loading or unloading?               		-----     -----------

29 CFR 1910.178 (k) (1)

Are wheel stops or other recognized positive

protection provided to prevent railroad cars from

moving during loading or unloading?                    		-----     -----------

29 CFR 1910.178 (k) (2)

Are fixed jacks available to support a semi-trailer

and prevent upending during the loading or unloading

when the trailer is not coupled to a tractor?          		-----     -----------

29 CFR 1910.178 (k)( 3)

# Operator Training

Are only trained and authorized operators permitted

to operate a powered industrial truck?                 		-----     -----------

29 CFR 1910.178 (l)

# Truck Operations

Is it prohibited for a person to stand or pass

under the elevated portion of any truck, whether

loaded or empty?                                       			-----     -----------

29 CFR 1910.178 (m) (2)

Are unauthorized personnel prohibited from riding

on powered industrial trucks?                          		-----     -----------

29 CFR 1910.178 (m) (3)

Is it prohibited for arms or legs to be placed

between the uprights of the mast or outside the

running lines of a truck?                              -			----     -----------

29 CFR 1910.178 (m) (4)

Is it required for load-engaging means to be fully

lowered, controls neutralized, power shut off, and

brakes set when a powered industrial truck is left

unattended?                                            			-----     -----------

29 CFR 1910.178 (m) (5) (i)

Is it required to maintain a safe distance from

the edge of ramps or platforms while on any

elevated dock, platform, or freight car?               		-----     -----------

29 CFR 1910.178( m) (6)

Is an overhead guard used as protection against

falling objects?                                      			 -----     -----------

29 CFR 1910.178 (m) (9)

Is a load backrest extension used whenever necessary

to minimize the possibility of the load or part of

it from falling backward?                              			-----     -----------

29 CFR 1910.178 (m) (10)

Are only approved industrial trucks used in

hazardous locations?                                   			-----     -----------

29 CFR 1910.178 (m) (11)

# Traveling

Is it required that all traffic regulations be

observed, including authorized plant speed limits?     	-----     -----------

29 CFR 1910.178 (n) (1)

Is it required to yield the right of way to

ambulances, fire trucks, or other vehicles

in emergency situations?                               		-----     -----------

29 CFR 1910.178 (n) (2)

Is it required that drivers not pass other trucks

traveling in the same direction at intersections,

blind spots, or other dangerous locations?             		-----     -----------

29 CFR 1910.178 (n) (3)

Is it required that drivers slow down and sound the

horn at cross aisles and other locations where

vision is obstructed?                                  			-----     -----------

29 CFR 1910.178 (n) (4)

Is it required that railroad tracks shall be

crossed diagonally, wherever possible?                		 -----     -----------

29 CFR 1910.178 (n) (5)

Is it required that when ascending or descending

grades that exceed 10 percent loaded trucks be

driven with the load upgrade?                          		-----     -----------

29 CFR 1910.178(n) (7) (i)

Is it required that on all grades the load and

load-engaging means be tilted back,

if applicable, and raised only as far as necessary

to clear the road surface?                             			-----     -----------

29 CFR 1910.178 (n) (7)( iii)

Is it required that under all travel conditions

the truck be operated at a speed that will permit

it to stop in a safe manner?                           			-----     -----------

29 CFR 1910.178 (n) (8)

Is stunt driving and horseplay prohibited?             		----     -----------

29 CFR 1910.178 (n) (9)

Are dockboards or bridge plates properly secured

before they are driven over?                           		-----     -----------

29 CFR 1910.178 (n) (11)

Is it required that elevators be approached slowly,

and then entered squarely after the elevator car

is properly leveled?                                   			-----     -----------

29 CFR 1910.178 (n) (12)

Is it required that motorized hand trucks enter

elevators or other confined areas with load end

forward?                                               			-----     -----------

29 CFR 1910.178 (n) (13)

# Loading

Are drivers instructed that only stable or safely

arranged loads be handled?                            		 -----     -----------

29 CFR 1910.178 (o) (1)

Are drivers instructed that only loads within the

rated capacity of the truck shall be handled?          		-----     -----------

29 CFR 1910.178 (o) (2)

Is a load-engaging means placed under the load as

far as possible?                                       			-----     -----------

29 CFR 1910.178 (o) (5)

Are drivers required to use extreme care when

tilting the load forward or backward, particularly

when high tiering?                                     			-----     -----------

29 CFR 1910.178 (o) (6)

# Operation of the Truck

Are personnel instructed that fuel tanks not be

filled while the engine is running?                    		-----     -----------

29 CFR 1910.178 (p) (2)

Is it required that spillage of oil or fuel be

carefully washed away or completely evaporated and

the fuel tank cap replaced before restarting the

engine?                                                			-----     -----------

29 CFR 1910.178 (p) (3)

Is it prohibited for a truck to be operated with

a leak in the fuel system until the leak

has been corrected?                                    			-----     -----------

29 CFR 1910.178 (p) (4)

Is it prohibited for open flames to be used for

checking electrolyte level in storage batteries or

gasoline level in fuel tanks?                         			-----     -----------

29 CFR 1910.178 (p) (5)

# Maintenance of Industrial Trucks

Is it required that no repairs be made in Class I,

II, and III locations?                                 			-----     -----------

29 CFR 1910.178 (q)  (2)

Is it required that repairs to the fuel and

ignition systems of industrial trucks, which involve

fire hazards, be conducted only in locations

designated for such repairs?                          		-----     -----------

29 CFR 1910.178 (q) (3)

Is it required that trucks in need of repairs to

the electrical system have the battery disconnected

before such repairs are made?                         		-----     -----------

29 CFR 1910.178 (q) (4)

Is it required that industrial trucks not be altered

without the manufacturer's approval?                   		-----     -----------

29 CFR 1910.178 (q) (6)

Is it required that industrial trucks be examined

before being placed in service?                        		-----     -----------

29 CFR 1910.178 (q) (7)

Is it required that water mufflers be filled daily

or as frequently as necessary to prevent depletion

of the water supply below 75 percent of the filled

capacity?                                              			-----     -----------

29 CFR 1910.178 (q) (8)

Is it required that vehicles with mufflers and

screens or other parts that may become clogged not

be operated while such screens or parts are clogged?   	-----     -----------

29 CFR 1910.178 (q) (8)

Is it required that any vehicle that emits hazardous

sparks or flames from the exhaust system be

immediately removed from service and not returned to

service until the cause for the emission of such

sparks and flames has been eliminated?                 		-----     -----------

29 CFR 1910.178 (q) (8)

Is it required that when the temperature of any

part of any truck is found to exceed its normal

operating temperature, thus creating a hazardous

condition, the vehicle be removed from service and

not be returned to service until the cause for such

overheating has been eliminated?                       		-----     -----------

29 CFR 1910.178 (q) (9)