- SAMPLE

- CHEMICAL HYGIENE PLAN

- Prepared for

(Organization Name)

- Prepared by: _____________________________ Date: ____________________

This Chemical Hygiene Plan (CHP) is intended to:

1.	Inform laboratory employees of the potential health and safety hazards present in their workplace.

2.	Inform laboratory employees of the precautions and preventive measures that have been established by this organization to protect employees from a workplace illness or injury.

3.	Inform laboratory employees of the required safety rules and procedures established by this organization to meet the requirements of 29 CFR 1910.1450 and 29 CFR 1910.1200.

The Chemical Hygiene Plan will be available for review upon request to all employees. Copies of the plan will be located in the following areas:

- Locations:

1.

2.

3.

4.

This CHP will be reviewed annually by (insert name or position), and updated as necessary. (insert name) is designated as the Chemical Hygiene Officer (See Sections VI and VII for details).

I.	STANDARD OPERATING RULES AND PROCEDURES

In addition to the specific laboratory standard operating procedures, the following general requirements are mandatory at all times:

- Routes of emergency egress shall be clearly indicated and unobstructed;

- All fire extinguishers shall remain functional and accessible;

- Access to the laboratory shall be restricted to authorized personnel only;

- Working in the lab alone is prohibited;

- Safety glasses and a lab apron/coat are mandatory at all times in the laboratory;

- Avoid all skin exposures to hazardous chemicals;

- Other personal protective equipment shall be used as directed by the CHO;

- Safety instruction signs, warning signs and exit signs shall be utilized and maintained in legible condition;

- Smoking, food, and beverages are prohibited in the laboratory at all times;

- Good housekeeping procedures shall be conducted daily;

- Counter tops and work benches shall be maintained clean, neat and orderly;

- If an incidental spill occurs, clean it up immediately;

- If a significant spill or leak occurs, the premises shall be vacated immediately or the emergency response plan shall be instituted;

- Material Safety Data Sheets received shall be submitted to the CHO;

- No manufacturer's label shall be removed or defaced from the original container;

- Identifying labels shall be utilized on all successive containers;

- Breakable containers shall be transported within a compatible, unbreakable, secondary container;

- Equipment which is damaged or malfunctioning shall not be used, particularly chipped glassware;

- Electrical equipment shall be maintained in good condition;

- Compressed gas cylinders shall be secured in an upright position;

- Pipetting by mouth suction is strictly prohibited;

- Procedures which are new or unfamiliar shall be referred to the CHO;

- Carcinogens shall only be utilized only at the direction of the CHO;

- Continuous inventory records shall be maintained on all chemicals;

- Chemical stock shall be rotated so that the shelf-life is not exceeded;

- Incompatible chemicals shall be segregated from each other;

- Chemical disposal shall be in accordance with all applicable laws and regulations;

- Flammable liquids shall be stored in a flammable storage cabinet;

- Laboratory hoods shall be utilized for all substances with an exposure level of 50 parts per million (ppm) or less, or unknown exposure limits or carcinogens;

- Laboratory hoods shall maintain a capture velocity of 100 linear feet per minute at the face of the hood;

- Hood usage and imcompatible chemicals shall be segregated;

- Hoods shall not be utilized for storage purposes;

- The CHO shall determine the adequacy of all lab hoods;

- All personnel shall wash their hands prior to entering and leaving the laboratory;

- Respirators shall be provided, maintained, used, and inspected in accordance with the Respiratory Protection Program;

- SCBA's or escape packs shall be readily available for emergency situations; and

- Inspections shall be conducted by the CHO at least monthly and documented.

Note: Section F of Appendix A of 1910.1450 lists additional safety recommendations from the book "Prudent Practices for Handling Hazardous Chemicals in Laboratories," National Academy Press, 1981.  These additional procedures should also be included as part of your chemical hygiene plan or as an appendix to to the plan if these hazards are present in your work place.

A.	Corrosive agents

B.	Electrically powered laboratory apparatus

C.	Fires, explosions

D.	Low temperature procedures

E.	Pressurized and vacuum operations

F.	Compressed gases

G.	Chemical Storage

H.	Decontamination

II.	CONTROL MEASURES TO REDUCE EMPLOYEE EXPOSURE TO HAZARDOUS CHEMICALS

A.	The following operations must be performed in Laboratory Fume Hoods:

B.	The following operations must be performed in Biological Safety Cabinets:

C.	The following operations must be performed in Glove Boxes:

D.	Respirators will be used in accordance with the respiratory protection program developed (insert organization’s name), and with the COMM/OSHA Respiratory Protection Standard 29 CFR 1910.134.  This program is available for employee review at (insert location).

E.	Appropriate personnel protective apparel and equipment compatible with the necessary degree of protection for substances handled will be provided and used in accordance with the COMM/OSHA Personanl Protective Equipment Standard 29 CFR 1910. 332.  (insert position title or name of the individual) will advise employees on the proper use of gloves, gowns, eye protection, barrier creams, etc.  Permeability charts are available for employee review at (insert location).

F.	Employees will be instructed on the location and proper use of eye wash stations and safety showers.  (insert position title or name of the individual) is responsible for this instruction.

G.	Employees will be trained (insert how often, e.g., annually) on the use of fire extinguishers and other fire protection systems.  (insert position title or name of the individual) is responsible for this instruction.

III.	MAINTENANCE OF FUME HOODS AND OTHER PROTECTIVE EQUIPMENT

A.	Fume hoods will be inspected every 		 months by (insert position title or name of the individual).  The adequacy of face velocity will be determined by (insert method).   Reports of hood inspections will be available for employee review at (insert location).

B.	Biological Safety Cabinets: will be inspected and filters replaced every

month by (insert position title or name of the individual).

(Repeat the above for each additional major category of protective equipment, such as ventilation systems for storage cabinets. safety showers and eyewash stations, etc., indicating how often they are inspected, by whom, what is measured, and where the inspection records and checklists are filed.)

IV.	EMPLOYEE INFORMATION AND TRAINING

A.	All employees covered by this CHP will be provided with information and training to ensure that they are apprised of the hazards of chemicals present in their work area.  This training will be given at the time of initial assignment and prior to new assignments involving different exposure situations.

- Refresher training will be given (insert how often).

B.	Information

Employees will be informed of:

1.	The contents of 1910.1450 and its appendices.  A copy of the standard will be available to employees for review at (insert location).

2.	The availability and location of the written Chemical Hygiene Plan.

3.	The Permissible Exposure Limits (PELs) for the substances regulated by OSHA or the Threshold Limit Values (TLVs) established by ACGIH for other hazardous chemicals where there is no applicable OSHA standard.

4.	Signs and symptoms associated with exposures to hazardous chemicals used in the laboratory.

5.	Location and availability of known reference material on the hazards, safe handling, storage, and disposal of hazardous chemicals found in the laboratory including, but not limited to, material safety data sheets received from chemical suppliers.

C.	Training

Employee training will include:

- The methods and observations that may be used to detect the presence or release of a hazardous chemical.  These may include monitoring conducted by the employer, continuous monitoring devices, visual appearance or odor of hazardous chemicals when being released, etc.

2.	The physical and health hazards of chemicals in the work area.

3.	The measures employees can take to protect themselves from these hazards.  Measures should include specific procedures the employer has implemented to protect employees from exposure to hazardous chemicals such as, standard operating procedures, work practices, emergency procedures, and personal protective equipment to be used.

The training will be conducted by (insert position title or name of the individual).  The training sessions will consist of (insert training methods, e.g. videotape, slide tape, lecture, hands-on exercises, etc.). (See attached outline of a suggested training program).

D.	Each employee will be required to sign the training completion form documenting that he/she has received training.

V.	PRIOR APPROVAL FOR SPECIFIC LABORATORY OPERATIONS

Certain laboratory procedures that present a serious chemical hazard require prior approval by (insert name of position or individual) before work can begin.  For this facility.  These procedures include:

A.	Work with select carcinogens

B.	Work with reproductive hazards

C.	Work with neurotoxins

D.	Work with unknown chemicals

E.	Work using new procedures

F.	Work with excessively large quantities of hazardous chemicals

G.	Other

These chemicals include:

(insert a list of the acutely hazardous chemicals, for example benzene, 	cyanide, dinitrobenzene, ethylene oxide, mercury, etc. used in your 	laboratory)

Note: If your laboratory does no utilize these classes of chemicals, then include a sentence that states "Our laboratory does not at this time use any chemicals that are sufficiently hazardous to require prior approval before they are used”.

VI.	MEDICAL CONSULTATION AND EXAMINATION

(insert physician's or clinic’s name) shall provide medical consultation to all affected employees.  Medical attention, including follow-up examinations that the examining physician deems is necessary, will be provided under the following circumstances:

A.	Whenever an employee develops signs and symptoms associated with a hazardous chemical to which they may have been exposed, the employee shall be provided an opportunity to receive appropriate medical examination.  The employee shall contact the chemical hygiene officer to initiate the medical program.

B.	Where exposure monitoring reveals an exposure level routinely above the OSHA action level (or in the absence of an action level, exposure above the OSHA permissible exposure level (PEL)) for OSHA regulated substances for which there are medical monitoring and medical surveillance requirements, medical surveillance shall be established for that employee.

Currently our laboratory uses:

1.	(e.g. benzene)

2.	(e.g. formaldehyde)

3.	(list other substances covered which have a separate OSHA standard with medical surveillance requirements)

Note: If none of these substances is used, indicate that no substances for which OSHA has medical monitoring requirements are being used.

C.	Whenever an event takes place in the work area, such as a spill, leak, explosion or other occurrence resulting in the likelihood of a hazardous exposure, the affected employee shall be provided an opportunity for a medical consultation.  This consultation will be for the purpose of determining the need for a medical examination.

D.	All medical examinations and consultations will be performed by

(insert physician's name) or at (insert clinic/hospital name).

Note: All aspects of these examinations and consultations must to be performed by a licensed physician, or under the direct supervision of a licensed physician.  These examinations must be provided without cost to the employee, without loss of pay and at a reasonable time and place.

E.	(Insert name of position or individual, e.g., chemical hygiene officer) will provide the following information to the physician:

1.	Identity of the hazardous chemical to which the employee may have been exposed.

2.	A description of the conditions of the exposure including exposure date if available.

3.	A description of signs and symptoms of exposure that the employee is experiencing (if any).

F.	The physician's written opinion for an examination or consultation should include:

1.	Recommendations for future medical follow-up.

2.	Results of examination and associated tests.

3.	Any medical condition, revealed in the course of the examination, which may place the employee at increased risk as the result of exposure to hazardous chemicals in the workplace.

4.	A statement that the physician has informed the employee about the results 	of the consultation or medical examination and any medical conditions 	that may require additional examination or treatment.

G.	The medical results returned to (insert organization’s name) by the physician shall not include specific findings and diagnoses that are unrelated to occupational exposure.

VII.	RESPONSIBILITIES UNDER THE CHEMICAL HYGIENE PLAN

A.	(Insert name of position or individual) is designated as the Chemical Hygiene Officer for (insert your organization’s name).  Note:  Proper qualifications for this individual are very important and should include a background in chemistry and safety.

- The Chemical Hygiene Officer (CHO) will be responsible for:

a)   Working with administrators and other employees to develop and implement appropriate chemical hygiene policies and practices.

b)   Monitoring procurement, use, and disposal of chemicals used in the lab.

c)   Seeing that appropriate audits are maintained.

d)   Helping project directors develop precautions and adequate facilities.

e)   Knowing the current legal requirements concerning regulated substances.

f)   Seeking ways to improve the chemical hygiene program.

- Laboratory supervisors will be reponsible for:

a)   Ensuring that workers know and follow the chemical hygiene rules, that protective equipment is available and in working order, and that appropriate training has been provided.

b)   Providing regular, formal chemical hygiene and housekeeping inspections including routine inspections of emergency equipment.

c)   Knowing the current legal requirements concerning regulated substances.

d)   Determining the required levels of protective apparel and equipment.

- Ensuring that facilities and training for use of any material being ordered are adequate.

D. 	Laboratory worker is responsible for:

a)   Planning and conducting each operation in accordance with the institutional chemical hygiene procedures.

b)   Developing good personal chemical hygiene habits.

- The Chemical Safety Committee ( if applicable) will be responsible for:

- Providing continuing support for institutional chemical hygiene

- Ensuring that OSHA standards are met and that the chemical hygiene program is being followed.

- Ensuring health and safety in the laboratory is maintained.

- Discussing/updating the chemcial hygiene program whenever the standard is updated or new chemicals are introduced in the workplace.

VIII.	ADDITIONAL PROTECTION FOR WORK WITH SELECT CARCINOGENS, REPRODUCTIVE TOXINS, AND CHEMICALS WITH HIGH ACUTE TOXICITY

When any select carcinogens, reproductive toxins and chemicals with high acute toxicity are used, the following provisions will be employed where appropriate:

A.	Establishment of a designated area.

B.	Use of containment devices such as fume hoods or glove boxes.

C.	Procedures for safe removal of contaminated waste.

D.	Decontamination procedures.

Appendix ___ of the standard outlines the standard operating procedures that must be followed when working with these chemicals.

Note:  According to the standard, a "select carcinogen" means any substance which meets one of the following criteria:

A.	The substance is regulated by OSHA as a carcinogen.

B.	The substance is listed under the category, "known to be carcinogens," in the Annual Report on Carcinogens, published by the National Toxicology Program (NTP - latest edition).

C.	The substance is listed under Group I ("carcinogenic to humans") by the International Agency for Research on Cancer Monographs (IARC-latest edition).

D.	The substance is listed in either Group 2A or 2B by IARC, or under the category, "reasonably anticipated to be carcinogens" by NTP].

Special procedures to follow in this laboratory for the use of these chemicals are found in

Appendix 	 of this CHP.

- RECORDKEEPING

The CHO shall be responsible for maintaining all relevant records in accordance with the following:

- Training records shall be maintained for 3 years from the date of the training;

- Monitoring records shall be maintained for 30 years from the monitoring date; and

- Medical surveillance records shall be maintained for the duration of employment plus 30 years.

______________________________________ 			__________________

Chemical Hygiene Officer 				  	 Date

X.	EMERGENCY RESPONSE

Two additional COMM/OSHA standards interface with the chemical hygiene plan:

1.	20 CFR 1910.38 - Employee Emergency Plans and Fire Prevention Plans

2.	1910.120 (p) and (q) - Hazardous Waste Operations and Emergency Response (developed in response to SARA Title III).

Please review these two standards and develop appropriate emergency procedures for your facility is covered by one or both of these standards.

Appendix 	is our facility’s emergency action plan under 1910.38

Appendix 	is our facility’s emergency response plan under 1910.120

Note: This chemical hygiene plan was prepared by Michael F. Waxman, Ph.D. Department of Engineering, Professional Development College of Engineering, University of Wisconsin Madison, Wisconsin and adapted and issued (with permission) by the Bureau of State Risk Management.  This sample written program is available on computer disk.

Please note that this is a generic chemical hygiene plan that can be tailored for use in your facility. The CHP must be specific to your particular workplace and include the necessary work practices, procedures and policies to ensure that employees are protected from all potentially hazardous chemicals used in their work area.  Hazardous chemicals as defined by the standard include not only chemicals regulated in 29 CFR 1910, subpart Z, but also any chemical meeting the definition of a hazardous chemical with respect to health hazards as defined in the COMM/OSHA Hazard Communication Standard, 29 CFR 1910.1200(c).

- SAMPLE LABORATORY TRAINING PROGRAM

- I.	Occupational Exposure to Hazardous Chemicals in Laboratories Standard (29 CFR 1910.1450)

A.	Content of the standard and appendices.

B.	Location and explanation of the chemical hygiene plan.

C.	Location of reference materials and material safety data sheets (MSDS).

D.	Details of access to medical consultation and management system.

- II.	Physical Hazards

A.	Combustible liquids

B.	Compressed gases

C.	Explosive liquids and solids

E.	Organic peroxides

F.	Pyrophoric chemicals

G.	Unstable (reactive) chemicals

H.	Water-reactive materials

- III.	Health Hazards

A.	Local

1.	Irritants

2.	Corrosives

B.	Systematic

1.	Toxics

a.  Acute toxicity vs chronic toxicity

b.  Nervous system effects

c.  Respiratory system effects

d.  Reproductive system effects

2.	Sensitizers

3.	Carcinogens

- IV.	Route of Exposure

A.	Inhalation

B.	Skin absorption

C.	Ingestion

D.	Injection

- V.	Physical State and Amount of Absorption

A.	Gases/vapors

B.	Particulates

1.	Dust

2.	Mist

3.	Fume

- VI.	Methods to Reduce Exposure

A.	Work practices

B.	Personal hygiene

C.	Personal protective equipment

D.	Environmental controls

E.	Administrative controls

- VII.	Dose-Response Relationship

A.	Age

B.	Gender

C.	Body size

D.	Health status

E.	Personal habits

F.	Other exposures

- VIII.	Duration of Exposure

- IX.	Exposure Limits Including PELs and TLVs

A.	Definition

B.	Interpretation

C.	Use of collected data

- X.	Air Sampling

A.	Personal sampling required by OSHA

B.	Area monitoring

C.	Employee reports of illness

D.	Confined space work

E.	Emergency response

F.	Other

- XI.	Employee Concerns

A.	Symptoms -exposures

B.	Requirements for documentation

C.	Referrals

D.	Refusal to work

- XII.	Specific Standard Operating Procedures

- A.

- B.

- C.

- D.

**********************************

- SAMPLE

LABORATORY TRAINING PROGRAM

TRAINING DOCUMENTATION FORM

- NAME: 	 DATE:

- DEPARTMENT:

- TRAINER:

- EMPLOYEE NAME

- EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

- EMPLOYEE NAME

EMPLOYEE NAME

EMPLOYEE NAME

- SAMPLE

LABORATORY AUDIT FORM

Building & Room # 	Date:

- I.	Housekeeping:

Yes	No

Is the lab in a disorderly condition?

Is there evidence of chemical spills?

Are floors in need of a cleaning?

Are bench tops cluttered with unused equipment or lab ware?

Are the emergency shower/eyewash stations blocked?

Is there evidence of eating or drinking in the lab?

Are tripping hazards present?

Comments:

- II.	Chemical Use and storage

Yes	No

Have employees received Right to Know training for the chemicals in use?

Are chemicals stored according to hazard class?

Are secondary containers labeled with identity & hazard class information?

Are copies of the MSDSs readily available for the chemicals in use?

Are outdated chemicals kept beyond their usefulness?

Are refrigerators/freezers properly labeled/used for the storage of flammables

Are shelves/cabinets for chemical storage in good condition?

Are flammables stored in the flammable storage cabinet.

Are acids and bases stored properly

Are volatile chemicals with PELs < 100 ppm restricted to use in the hood?

Are compressed gas cylinders properly secured and labeled?

Comments:

List the type and the number of gallons of flammable solvents stored outside of a flammable solvent cabinet:

Acetone (1A)	Ethyl Ether (1A)

Hexane (1A)	Pentane (1A)

Methanol (1A)	Toluene (1B)

Note any instances of incompatible storage:

- III.	Hazardous wastes

A.	Hazardous Waste generated in this area:

| Waste Description | Generation Rate (units) |
| --- | --- |

Yes	No

Have all process been evaluated for proper waste disposal procedures?

Do procedures indicate a proper route of disposal for all product?

Are wastes transferred to the proper containers?

Are waste containers properly labeled as to their contents and hazards?

Are containers of wastes properly stored in the satellite area?

Comments:

- IV.	Personal Protective equipment

- Are staff using/wearing the following personal protective equipment?

Yes	No

Lab Coat?

Safety Glasses

Safety Goggles when necessary

Gloves compatible with the chemicals in use?

Apron

Respirator

Are lab coats/gloves removed before leaving the lab?

Comments:

- V.	HOODS

Yes	No

Are hoods cluttered with material or equipment?

Are hoods being used to store chemicals that are not in use?

Have the hoods been recently calibrated (face velocity)?

Are the hoods being used properly by staff.?

Are the slots/baffles blocked by equipment or chemicals?

Are equipment/processes placed 6 inches behind the face of the hood?

Comments:

- VI.	Emergency Procedures

Yes	No

Have staff been trained in emergency procedures?

Are emergency procedures posted?

Are emergency response phone numbers displayed near the phone

Are exits marked?

Are spill cleanup procedures and kits available for all materials in use:

Yes	No

Solvents

Acids/Bases

Broken Glass/sharps

Other (list)

Are emergency shower/eyewash stations readily available?

Are appropriate fire extinguishers readily available?

Has the fire extinguisher been recently maintained

Is there a first aid kit available in the room

Is there a fire blanket in the area?

Comments:

- VII.	Equipment

Yes	No

Is equipment used within its designed purpose?

Is apparatus properly secured/supported?

Are fail safes in use when possible?

Are all water/glassware connections secured?

Do electrical plugs or cords show evidence of:

Yes	No

frayed cords

overloaded cords (warm to the touch)

altered or damaged plugs (ground removed)

extension cords in use

Comments:

- VIII.	Standard Operating Procedures

Yes	No

Are SOPs available for the process(s) being conducted in the lab?

Are the SOPs reviewed or updated with new information?

Are the staff following the SOPs as written?

Comments:

- BIOSAFETY

Source(s) of infectious waste:

Yes	No

Used needles are bent, not broken?

All sharps are placed in labeled puncture-resistant containers?

Are hands washed after glove removal/hand contact with infectious agents

Is PPE removed before leaving the work area.

Is aerosolation, splashing or spraying kept to a minimum-

Is eating, smoking or drinking prohibited in the labs.

Is the appropriate PPE available and in use?

Is the area posted with a BIOHAZARD symbol & name of the infectious agent,

Are warning labels affixed to containers of infectious waste?

Have staff received training to work with infectious waste?

Are work surfaces decontaminated after procedures, spills and at the end of the shift.

How is the infectious waste disposed of:

Comments:

Source:  U W Superior

- HOOD PERFORMANCE EVALUATION

Hood Location: Bldg. & Room	Hood Number

List any dedicated uses of the hood:

- CURRENT PHYSICAL CONDITION

Yes	No

Is the hood used for storage of equipment or chemicals?

Are the slots blocked by objects?

Are processes/equipment located within 6 inches of the sash or rear slots?

Is the bench area inside the hood suffering from poor housekeeping?

Is large equipment in use in the hood that is blocking air flow?

Are open chemical containers (not in use) present?

Do any large heated processes/equipment affect the air flow in the hood?

Is the hood monitor inoperative or inaccurate?

Does the sash move with any difficulty?

Is the sash glass cracked, fogged, or in need of repairs?

Is the light/fixture in need of repairs?

Are hood fixtures (gas, water, air, etc.) corroded, dirty or in need of repair?

Description/Comments:

- Any of the above conditions answered with a "yes" should be corrected immediately.

- SUMMARY OF INSPECTION

Are repairs needed to the system? If so, list:

Repair	Date Required

- Does the hood monitor readout correspond with the measured face velocity?

- Does the face velocity meet health and safety requirements?

- Has the face velocity changed ± 10% since the last evaluation?

- If so, explain

Recommendations:

- HOOD PERFORMANCE EVALUATION

- FACE VELOCITY MEASUREMENTS

Hood Location:  Bldg. and Room 	Hood Number

Date of Evaluation 	Date of Last Evaluation

- Name of Evaluator

Instrument: 	S/N 	Calib. date:

Indicate the position of any fixed equipment in the hood:

- Full Face

1.	Divide the hood face (sash wide open) into a grid with 9-12 equal sections.

2.	Measure the air velocity in the center of each grid.  Record in the grid diagram labeled as "full face".

3.	Determine the mean face velocity and report in the space below.

4.	Use smoke tubes to determine air flow direction at the hood face and air flow patterns within the hood.  Record unusual flow patterns in the grid diagram.

# Operating Sash Height

If it is necessary to operate the hood with the sash at a lower height to achieve the required face velocity of 100 lfpm, adjust the sash to the appropriate height and repeat steps 1-4 above.  Record the face velocities on the grid marked "operating height".  Indicate the operating point on the hood face.

| Full Face |  |  |  |  |  |  |  | Operating Height |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| RESULTS: | "Full Face" |  | "Operating Height" |
| --- | --- | --- | --- |
| Hood Face dimensions (W X H): |  |  |  |
| Grid dimensions (W X H) |  |  |  |
| Mean Face Velocity (fpm): |  |  |  |
| Record the Hood Monitor Set Point: |  |  |  |
| Was a hood performance report posted on the hood? Yes/No |  |  |  |  |

Recommendations:

LABORATORY SAFETY CHECK-LIST        ______________________________________________

______________________________________________

______________________________________________

(Installation, Address and Date)

|  | Yes | No |  |  | Yes | No |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Fire Extinguisher Access |  |  |  | 37. Secondary Containers |  |  |
| 2. Fire Extinguisher Inspection |  |  |  | 38. Broken Glassware |  |  |
| 3. Exit Pathways |  |  |  | 39. Waste Disposal Containers |  |  |
| 4. Exit Signs & Lighting |  |  |  | 40. Electrical Grounding |  |  |
| 5. Exit Doors |  |  |  | 41. Elec. Panel Access |  |  |
| 6. First Aid Supplies |  |  |  | 42. Elec. Labeling & I.D. |  |  |
| 7. Fire Blanket |  |  |  | 43. Elec. Cord Strain Relief |  |  |
| 8. Fire Alarms |  |  |  | 44. Elec. Wiring / Device Defects |  |  |
| 9. SCBA's / Escape Packs |  |  |  | 45. Elec. Classification Hazards |  |  |
| 10. Respirator Use & Storage |  |  |  | 46. Elec. Fail Safe Conditions |  |  |
| 11. Emergency Equipment |  |  |  | 47. Elec. Extension Cords |  |  |
| 12. Housekeeping / Sanitation |  |  |  | 48. Training Deficiencies |  |  |
| 13. Tripping Hazards |  |  |  | 49. Procedural Deficiencies |  |  |
| 14. Spills & Leaks |  |  |  | 50. Defective Equipment Tags |  |  |
| 15. Material Storage |  |  |  | 51. Personal/Area Monitoring |  |  |
| 16. Flammable Storage Cabinets |  |  |  | 52. Signs & Posters |  |  |
| 17. Compressed Gas Cylinders |  |  |  | 53. WOHS 902 |  |  |
| 18. Material Labeling |  |  |  | 54. OSHA 200 (February) |  |  |
| 19. Container Labeling |  |  |  | 55.Emergency Phone Nos. |  |  |
| 20. MSDS's |  |  |  |  |  |  |
| 21. Stair Condition |  |  |  | CORRECTIVE ACTIONS & DATES: |  |  |
| 22. Stair Handrails |  |  |  |  |  |  |
| 23. Ladder Inspections |  |  |  |  |  |  |
| 24. Platform Guardrails |  |  |  |  |  |  |
| 25. Eye Wash Stations |  |  |  |  |  |  |
| 26. Drench Showers |  |  |  |  |  |  |
| 27. Safety Glasses |  |  |  |  |  |  |
| 28. Safety Goggles |  |  |  |  |  |  |
| 29. Face Shields |  |  |  |  |  |  |
| 30. Lab Coats / Aprons |  |  |  |  |  |  |
| 31. Appropriate Gloves |  |  |  |  |  |  |
| 32. Warning & Safety Signs |  |  |  |  |  |  |
| 33. General Ventilation |  |  |  |  |  |  |
| 34. Fume Hood Ventilation |  |  |  |  |  |  |
| 35. Chemical Segregation |  |  |  |  |  |  |
| 36. Stock Rotation |  |  |  |  |  |  |

Definitions

Action level means a concentration designated in 29 CFR part 1910 for a specific substance, calculated as an eight (8)-hour time-weighted average, which initiates certain required activities such as exposure monitoring and medical surveillance. 

Assistant Secretary means the Assistant Secretary of Labor for Occupational Safety and Health, U.S. Department of Labor, or designee. 

Chemical Hygiene Officer means an employee who is designated by the employer, and who is qualified by training or experience, to provide technical guidance in the development and implementation of the provisions of the Chemical Hygiene Plan.

Chemical Hygiene Plan means a written program developed and implemented by the employer which sets forth procedures, equipment, personal protective equipment and work practices that are capable of protecting employees from the health hazards presented by hazardous chemicals used in that particular workplace.

Combustible liquid means any liquid having a flashpoint at or above 100 deg. F (37.8 deg. C), but below 200 deg. F (93.3 deg. C), except any mixture having components with flashpoints of 200 deg. F (93.3 deg. C), or higher, the total volume of which make up 99 percent or more of the total volume of the mixture. 

Compressed gas means: 

(i) A gas or mixture of gases having, in a container, an absolute pressure exceeding 40 psi at 70 deg. F (21.1 deg. C); or 

(ii) A gas or mixture of gases having, in a container, an absolute pressure exceeding 104 psi at 130 deg. F (54.4 deg C) regardless of the pressure at 70 deg. F (21.1 deg. C); or 

(iii) A liquid having a vapor pressure exceeding 40 psi at 100 deg. F (37.8 C) as determined by ASTM D-323-72. 

Designated area means an area which may be used for work with "select carcinogens," reproductive toxins or substances which have a high degree of acute toxicity. A designated area may be the entire laboratory, an area of a laboratory or a device such as a laboratory hood. 

Emergency means any occurrence such as, but not limited to, equipment failure, rupture of containers or failure of control equipment which results in an uncontrolled release of a hazardous chemical into the workplace. 
Employee means an individual employed in a laboratory workplace who may be exposed to hazardous chemicals in the course of his or her assignments. 

Explosive means a chemical that causes a sudden, almost instantaneous release of pressure, gas, and heat when subjected to sudden shock, pressure, or high temperature. 

Flammable means a chemical that falls into one of the following categories: 

(i) Aerosol, flammable means an aerosol that, when tested by the method described in 16 CFR 1500.45, yields a flame protection exceeding 18 inches at full valve opening, or a flashback (a flame extending back to the valve) at any degree of valve opening; 

(ii) Gas, flammable means: 

(A) A gas that, at ambient temperature and pressure, forms a flammable mixture with air at a concentration of 13 percent by volume or less; or 

(B) A gas that, at ambient temperature and pressure, forms a range of flammable mixtures with air wider than 12 percent by volume, regardless of the lower limit. 

(iii) Liquid, flammable means any liquid having a flashpoint below 100 deg F (37.8 deg. C), except any mixture having components with flashpoints of 100 deg. C) or higher, the total of which make up 99 percent or more of the total volume of the mixture. 

(iv) Solid, flammable means a solid, other than a blasting agent or explosive as defined in § 1910.109(a), that is liable to cause fire through friction, absorption of moisture, spontaneous chemical change, or retained heat from manufacturing or processing, or which can be ignited readily and when ignited burns so vigorously and persistently as to create a serious hazard. A chemical shall be considered to be a flammable solid if, when tested by the method described in 16 CFR 1500.44, it ignites and burns with a self-sustained flame at a rate greater than one-tenth of an inch per second along its major axis. 

Flashpoint means the minimum temperature at which a liquid gives off a vapor in sufficient concentration to ignite when tested.

Hazardous chemical means a chemical for which there is statistically significant evidence based on at least one study conducted in accordance with established scientific principles that acute or chronic health effects may occur in exposed employees. The term "health hazard" includes chemicals which are carcinogens, toxic or highly toxic agents, reproductive toxins, irritants, corrosives, sensitizers, hepatotoxins, nephrotoxins, neurotoxins, agents which act on the hematopoietic systems, and agents which damage the lungs, skin, eyes, or mucous membranes.  

Laboratory means a facility where the "laboratory use of hazardous chemicals" occurs. It is a workplace where relatively small quantities of hazardous chemicals are used on a non-production basis. 

Laboratory scale means work with substances in which the containers used for reactions, transfers, and other handling of substances are designed to be easily and safety manipulated by one person. 

Laboratory-type hood means a device located in a laboratory, enclosure on five sides with a movable sash or fixed partial enclosed on the remaining side; constructed and maintained to draw air from the laboratory and to prevent or minimize the escape of air contaminants into the laboratory; and allows chemical manipulations to be conducted in the enclosure without insertion of any portion of the employee's body other than hands and arms. 
 
Laboratory use of hazardous chemicals means handling or use of such chemicals in which all of the following conditions are met: 

(i) Chemical manipulations are carried out on a "laboratory scale;" 

(ii) Multiple chemical procedures or chemicals are used; 

(iii) The procedures involved are not part of a production process, nor in any way simulate a production process; and 

(iv) "Protective laboratory practices and equipment" are available and in common use to minimize the potential for employee exposure to hazardous chemicals. 

Medical consultation means a consultation which takes place between an employee and a licensed physician for the purpose of determining what medical examinations or procedures, if any, are appropriate in cases where a significant exposure to a hazardous chemical may have taken place. 

Organic peroxide means an organic compound that contains the bivalent -O-O- structure and which may be considered to be a structural derivative of hydrogen peroxide where one or both of the hydrogen atoms has been replaced by an organic radical. 

Oxidizer means a chemical other than a blasting agent or explosive as defined in § 1910.109(a), that initiates or promotes combustion in other materials, thereby causing fire either of itself or through the release of oxygen or other gases. 

Physical hazard means a chemical for which there is scientifically valid evidence tat it is a combustible liquid, a compressed gas, explosive, flammable, an organic peroxide, an oxidizer pyrophoric, unstable (reactive) or water-reactive. 

Protective laboratory practices and equipment means those laboratory procedures, practices and equipment accepted by laboratory health and safety experts as effective, or that the employer can show to be effective, in minimizing the potential for employee exposure to hazardous chemicals. 

Reproductive toxins means chemicals which affect the reproductive chemicals which affect the reproductive capabilities including chromosomal damage (mutations) and effects on fetuses (teratogenesis). 

Select carcinogen means any substance which meets one of the following criteria: 

(i) It is regulated by OSHA as a carcinogen; or 

(ii) It is listed under the category, "known to be carcinogens," in the Annual Report on Carcinogens published by the National Toxicology Program (NTP)(latest edition); or 

(iii) It is listed under Group 1 ("carcinogenic to humans") by the International Agency for research on Cancer Monographs (IARC)(latest editions); or 

(iv) It is listed in either Group 2A or 2B by IARC or under the category, "reasonably anticipated to be carcinogens" by NTP, and causes statistically significant tumor incidence in experimental animals in accordance with any of the following criteria: 

(A) After inhalation exposure of 6-7 hours per day, 5 days per week, for a significant portion of a lifetime to dosages of less than 10 mg/m(3); 

(B) After repeated skin application of less than 300 (mg/kg of body weight) per week; or 

(C) After oral dosages of less than 50 mg/kg of body weight per day. 

Unstable (reactive) means a chemical which is the pure state, or as produced or transported, will vigorously polymerize, decompose, condense, or will become self-reactive under conditions of shocks, pressure or temperature. 

Water-reactive means a chemical that reacts with water to release a gas that is either flammable or presents a health hazard.