# SAFETY OPERATING PROCEDURE

Blower Vacuum - Electric

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

### PRE-OPERATIONAL SAFETY CHECKS

- Do not use an electric blower vacuum in wet conditions.

- Do not wear jewellery or loose clothing that can be drawn into the air inlet.

- Check machine for loose/missing nuts, bolts and screws. Tighten and/or replace as needed.

- Inspect the parts of the mulching cutting head (where fitted). Replace cutting line if worn or damaged

- Ensure the debris bag is in sound condition and is properly fitted with no gaps.

- Ensure all guards are fitted, secure and functional.

- Ensure the machine and extension cord are tagged and are current.

- Ensure a tagged RCD is used when operating the machine.

- Trip test RCD before operation.

- Ensure the extension cord is in good condition.

- The extension cord must be uncoiled as coiled cables can overheat.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Ensure no person or animal is endangered when operating equipment.

- Adjust the shoulder harness as required to ensure it is in the most comfortable position.

- Keep a firm grip. Hold handles with fingers together encircling handles.

- Maintain a straight wrist position. Avoid using your wrist in a bent, extended or twisted position.

- Maintain footing and balance at all times. Do not stand on slippery, uneven or unstable surfaces. Do not work in odd positions or on ladders.

- Select the mode indicator to adjust the nozzle position for blow or vacuum.

- Do not pull cable around sharp objects.

- Do not direct the blower in the direction of other people.

- Take wind conditions into account. Avoid open doors and windows.

- Minimize dust by using blower at lower speeds.

- If the blower vacuum starts to vibrate abnormally, switch off and check immediately.

- Switch off and disconnect from the electricity supply before removing or replacing the debris bag.

- Do not place hands in bag when emptying. Sharp debris can cause injury.

#### HOUSEKEEPING

- Remove any foreign material from motor air inlet and nozzle

- Keep the work area and implement shed in a clean and tidy condition.

## POTENTIAL HAZARDS

	Noise 			Electrical hazards			Flying debris			Dust