# SAFETY OPERATING PROCEDURES

Portable Air Compressor

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Ensure all flammable materials are safely stored. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Locate the compressor in a suitable location for safe operation.

- Lock the wheels on the base of the compressor to prevent movement.

- Check that all fittings and connections are in good condition prior to starting.

- Check all fittings are securely connected prior to being pressurised.

- Faulty equipment must not be used. Immediately report suspect machinery.

- Locate and ensure you are familiar with the operation of the ON/OFF starter.

#### OPERATIONAL SAFETY CHECKS

- Start the compressor noting pressure increase and cut-out/cut-in pressure.

- Listen for any air leaks from any flexible airlines and immediately report if any leaks are observed.

- Adjust pressure regulator to suit work requirements.

- Check the compressor at regular intervals.

#### HOUSEKEEPING

- Switch off machine.

- Leave the machine, hose and work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Unsecured hoses whipping under pressure

	Compressed air