# SAFETY OPERATING PROCEDURES

Drill Press

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspace and walkways to ensure no slip-hazards are present.

- Check that the drill chuck guard is in position.

- Ensure the chuck key (if used) has been removed from the drill chuck.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Follow correct clamping procedures to ensure work is secure.

- If the job obstructs the walkway erect a barricade.

- Adjust spindle speed to suit drill or cutter diameter.

- Faulty equipment must not be used. Immediately report suspect equipment.

##### OPERATIONAL SAFETY CHECKS

- Never leave the Drill Press while it is running.

- Before making adjustments or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

- Feed downwards at a sufficient rate to keep the drill cutting.

- Feed with care as the drill breaks through the underside of the work.

- Use a safe working posture (beware of hair catching).

##### HOUSEKEEPING

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

##### POTENTIAL HAZARDS

- Hair/clothing entanglement - rotating spindle/drill

	Eye injuries

- Flying swarf/chips

	Sharp edges & burrs