# SAFETY OPERATING PROCEDURES

Pedestal Grinder

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be used when using this machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip hazards are present.

- Ensure all guards and safety shields are in position before starting the grinder.

- Ensure that the wheels do not touch the work rest and that the gap between wheel and rest is no greater than 1.5mm.

- Check that wheels are running true and are not glazed or loaded.

- Locate and ensure you are familiar with the operation of the ON/OFF starter.

- Faulty equipment must not be used. Immediately report any suspect machinery.

OPERATIONAL SAFETY CHECKS

- Stand to the side of the wheels when starting up.

- Let the wheels gain maximum speed before starting to grind.

- Do not grind on the side of the wheel.

- Small objects must not be held by hand.

- Never leave the machine running unattended.

- Do not bend down near the machine whilst it is running.

- Never force the workpiece against a wheel.

- Slowly move the workpiece across the face of the wheel in a uniform manner.

- Coolant spilt on the floor should be immediately absorbed.

HOUSEKEEPING

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

## POTENTIAL HAZARDS

## 	Hot Metal 				Sparks			Noise					Sharp edges and burrs

## 	Entanglement			Wheels ‘run on’ after switching off		n	Eye injuries

FORBIDDEN

- Workpiece must never be held with gloves, cloth, apron or pliers

- Grinding non-ferrous metals