# SAFETY OPERATING PROCEDURES

Metal Lathe

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip hazards are present.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Ensure all guards are in place.

- Check that the job is clamped tight in the chuck.

- Remove all tools from the bed and slides of the machine.

- Ensure correct speed for machining process is selected.

- Remove the chuck key before starting the lathe.

- Do not try to lift chucks or face plates that are too heavy for you.

- Faulty equipment must not be used. Immediately report suspect machinery.

OPERATIONAL SAFETY CHECKS

- Never leave the lathe running unattended.

- Before making adjustments or measurements switch off and bring the machine to a complete standstill.

- Do not attempt to slow/stop the chuck or revolving work by hand.

- Avoid letting swarf build up on the tool or job. Stop the machine and remove it.

- Always remove the chuck key from the chuck.

- Do not store tools and parts on top of the machine.

## HOUSEKEEPING

- Switch off the machine and reset all guards to a fully closed position.

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

	Flying objects - chuck key left in chuck

	Cutting tool injury when cleaning, filing or polishing

	Rotating machine parts - entanglement

	Metal splinters/swarf

	Eye injuries