# SAFETY OPERATING PROCEDURES

Milling Machine

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Do not leave equipment on top of the machine.

- Check that machine guards are in position.

- Ensure cutter is in good condition and securely mounted.

- Check coolant delivery system to allow for sufficient flow of coolant.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Keep clear of moving machine parts.

- Never leave the machine running unattended.

- Follow correct clamping procedures- keep overhangs as small as possible and check work piece is secure.

- Set the correct speed to suit the cutter diameter, the depth of cut and the material.

- Before making adjustments and measurements or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

#### HOUSEKEEPING

- Switch off the machine.

- Remove milling cutters and store them safely.

- Leave the machine and work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Sharp cutters

	Moving components 	- hair/clothing entanglement

	Eye injury

	Skin irritation

	Metal splinters and burrs

	Flying debris