# SAFETY OPERATING PROCEDURES

Wood Lathe

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection may be required when using this machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip-hazards are present.

- Check safety guards are in position.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Ensure the workpiece has been suitably prepared for the lathe operation.

- Workpiece must be securely fastened to face plate, chuck or between centres.

- Adjust speed to suit the diameter of the work and turning operation.

- Rotate the workpiece by hand to check clearance between tool rest and bed.

- Ensure the cutting tools are sharp and in good condition.

- Start the dust extraction unit before using the machine.

- Faulty equipment must not be used. Immediately report suspect equipment.

#### OPERATIONAL SAFETY CHECKS

- Never leave the machine running unattended.

- Before making adjustments switch off and bring the machine to a complete standstill.

- Keep the tool rest adjusted close to the work and at the correct height.

- Adjust speed to suit the diameter of the work and turning operation.

- Stop the lathe and remove all tool rests before sanding.

#### HOUSEKEEPING

- Switch off the machine.

- Return all chisels and other tools to racks.

- Remove all tool-rests and place in rack.

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

- Rotating spindle/work - entanglement

- Eye injuries - flying debris - defects in timber

	Airborne dust