# SAFETY OPERATING PROCEDURES

Pan Brake

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Guards or safety devices shall never be removed or adjusted, except by an authorized person for maintenance purposes.

- Working parts should be well lubricated and the jaws and fingers free of rust and dirt.

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Be aware of other personnel in the immediate vicinity and ensure the area is clear before using equipment.

- Familiarise yourself with and check all machine operations and controls.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Never use pan brakes for bending metal that is beyond the machine’s capacity with respect to thickness, shape, or type.

- Never attempt to bend rod, wire, strap, or spring steel sheets in a pan brake.

- Remove the pan brake fingers that are in the way - use only the pan brake fingers required to make the bend.

- Ensure the pan brake fingers that are not removed for an operation are securely seated and firmly tightened before the machine is used.

- Ensure fingers and limbs are clear before operating the pan brake.

- Lower finger clamps to work - do not drop.

- Check workpiece is secure.

- Keep clear of moving counterweight (where fitted).

#### HOUSEKEEPING

- Lower finger clamps to a safe position.

- Return all accessories to storage racks.

- Leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

- Sharp edges and burrs

- Squash/crush and pinch points

- Impact from counterweight