# SAFETY OPERATING PROCEDURES

Cold Saw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure that no slip/trip hazards are present.

- Ensure saw blade is in good condition.

- Locate and check the operation of the ON/OFF starter.

- Check that all safety guards are in working order.

- Check the operation of the work vice.

- Check coolant delivery system to allow for sufficient flow of coolant.

- Faulty equipment must not be used. Immediately report suspect equipment.

OPERATIONAL SAFETY CHECKS

- Ensure that the work piece is securely held in the work vice.

- Support overhanging work and signpost if it presents a hazard.

- Never leave the machine running unattended.

- Attention must be paid to unusual noises during the sawing process.

- Never force the saw into the workpiece. Use a slow and even feed rate.

- Before making adjustments or before cleaning swarf accumulations switch off and bring the machine to a complete standstill.

- Immediately absorb any coolant spills.

#### HOUSEKEEPING

- Switch off the machine.

2.	Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

	Possible skin irritation from coolants

	Eye injuries

	Sharp edges and burrs - metal splinters

	Noise

FORBIDDEN

- Attempting to cut very small items

- Attempting to cut materials other than metal