# SAFETY OPERATING PROCEDURES

Horizontal Metal Bandsaw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection may be required for some operations. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Check that all guards are in position.

- Ensure hydraulic damping mechanism functions.

- Check that the blade is in good condition.

- Ensure that blade speed, blade tension and blade tracking are properly adjusted.

- Check coolant delivery system to allow for sufficient flow of coolant.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

8.	Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Lift the head of unit up and lock it in the upward position.

- Set the angle of the vice, or check it to ensure its squareness.

- Clamp work piece firmly into the vice. Long material must be supported.

- Adjust blade guards to cover unused portion of blade.

- Ensure hands are away from the blade, and then turn the machine on.

- Allow the upper head assembly to come down slowly until the teeth are cutting the material.

- Keep hands away from the point of operation during cutting.

- Turn off the machine and bring it to a complete standstill if the blade is to be lifted out of an uncompleted or jammed cut.

- Stop the machine and bring it to a complete standstill before removing scrap pieces from the vice area or making adjustments.

- Stop the saw immediately if the blade develops a ‘click’. Report it to your teacher.

- Ensure the cutting head is locked in the upward position before removing work piece from vice.

#### HOUSEKEEPING

- Switch off the saw and reset all guards to a fully closed position.

- Leave the machine in a safe, clean and tidy state.

## FORBIDDEN

## Pushing down on the cutting head while it is cutting

## Leaving the machine running unattended