# SAFETY OPERATING PROCEDURES

Manual Bending Rolls

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure all guards are in place, properly secured and functional.

- Guards or safety devices must never be removed or adjusted, except by an authorized person for maintenance purposes.

- Working parts should be well lubricated and the rolls free of rust and dirt.

- The area around the machine must be adequately lit and kept free of materials, which might cause slips or trips.

- Be aware of other personnel in the immediate vicinity and ensure the area is clear before using equipment.

- Familiarise yourself with and check all machine operations and controls.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Do not attempt to roll material beyond the capacity of the machine.

- Never attempt to bend rod or wire in bending rolls unless the rolls are grooved for this purpose.

- Adjust both ends of the rollers evenly.

- Take extreme care during the initial feeding of the workpiece into the rolls.

- The workpiece should always be held sufficiently far back from the edge being fed into the rolls to allow for the infeed speed of the machine.

- Wind handle at a slow even rate. Be aware of rotating rollers.

#### HOUSEKEEPING

- Ensure the handle is left in a safe position after use.

2.	Leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Sharp edges and burrs		Crush and pinch points		Entanglement

	Operator’s hands may be caught and drawn into the counter-rotating rollers