# SAFETY OPERATING PROCEDURES

Metal Cutting Guillotine

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure fixed guards are in place to prevent hands or other parts of the body from entering the trapping space.

- Guards or safety devices must never be removed or adjusted, except by an authorized person for maintenance purposes.

- Working parts should be well lubricated and free of rust and dirt.

- The area around the machine must be adequately lit and kept free of materials, which might cause slips or trips.

- Be aware of other personnel in the immediate vicinity and ensure the area is clear before using equipment.

- Familiarise yourself with and check all machine operations and controls.

- Ensure cutting table is clear of scrap and tools.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Do not attempt to cut material beyond the capacity of the machine.

- Never attempt to cut rod, strap or wire with this machine.

- Use correct lifting procedures when handling large sheets of material.

- Take extreme care during the initial feeding of the workpiece into the machine.

- The workpiece should always be held sufficiently far back from the edge being fed into the guillotine.

- Ensure fingers and limbs are clear before actuating the guillotine.

- Hold material firmly to prevent inaccurate cutting due to creep.

- When cutting ensure feet are positioned to avoid contact with the foot operated lever.

#### HOUSEKEEPING

- Remove all off cuts and place them in either in the storage rack or waste bin.

- Leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Sharp edges and burrs		Crush and pinch points		Handling metal sheets