# SAFE OPERATING PROCEDURES

Bandsaw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection may be required for some operations. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Check that all guards are in position.

- Ensure push stick is available.

- Lower the blade guide and guard to full effect.

- Start the dust extraction unit before using the saw.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

Keep fingers off the line of the cut at all times

- Never leave the machine running unattended.

- The work piece should be fed forward evenly and held firmly on the table to ensure effective control during cutting whilst keeping hands in a safe position.

- Use a push stick when feeding material past the blade.

- Do not force a wide blade on a cut of small radius. Use relief cuts when cutting sharp curves.

- Before making adjustments switch off the saw and bring the machine to a complete standstill.

- Stop the machine before attempting to back the work away from the blade.

- Stop the saw immediately if the blade develops a ‘click’. Report it to your teacher.

#### HOUSEKEEPING

- Switch off the saw and reset all guards to a fully closed position.

- Leave the machine in a safe, clean and tidy state.

## FORBIDDEN

## 	Attempting to cut very small items

	Cutting cylindrical or irregular stock