# SAFETY OPERATING PROCEDURES

Circular Bench Saw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure that no slip/trip-hazards are present.

- Check that all safety guards are in position and are set to the minimum clearance from the workpiece being cut.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Ensure all locks are securely tightened.

- Ensure table and work area is clear of all tools, off-cut timber and sawdust.

- Start the dust extraction unit before using the machine.

- Faulty equipment must not be used. Immediately report suspect equipment.

## OPERATIONAL SAFETY CHECKS

- Do not cut freehand.

- Allow the saw blade to obtain maximum speed before making a cut.

- Use push stick (at least 400 mm long) to guide timber through saw.

- Always stand to one side of the line of cut.

- Never remove off cuts or sawdust from the saw table while the saw is running.

- Before making adjustments switch off and bring the machine to a complete standstill.

- Never leave the machine running unattended.

- Remove the rip fence when using the mitre gauge.

- Make sure someone “tails out” when cutting long material.

HOUSEKEEPING

##### Switch off the machine.

##### Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

	Kickback - wood may catch or jam and be flung back violently

##### 	Airborne dust			Eye injuries		Contact with blade at point of operation

FORBIDDEN

	Cutting irregular stock, branches or wood with embedded nails or screws