# SAFETY OPERATING PROCEDURES

Abrasive Cut Off Saw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure the saw is properly secured to a worktable by bolts/ clamps at approximately hip height.

- Ensure the saw is operated on an RCD protected circuit.

- Use abrasive cut off wheels with the correct size arbor hole.

- Use abrasive cut off wheels which have a maximum safe operating speed greater than the “no load RPM” marked on the machine’s nameplate.

- Before each use inspect the cut off wheel for chips and cracks.

- Check workspaces and walkways to ensure no slip/trip-hazards are present.

- Ensure all guards are in place, in working order, and in proper adjustment and alignment.

- Ensure the depth stop is properly adjusted.

- Ensure you are familiar with the operation of the ON/OFF starter.

- Keep table and work area clear of all tools and off-cut material.

- Faulty equipment must not be used. Immediately report suspect equipment.

OPERATIONAL SAFETY CHECKS

This machine and abrasive wheel is not intended to cut wood or wood products.

- Ensure all adjustments to machine are secure before making a cut.

- Do not hold a workpiece by hand. Workpiece will become very hot while being cut.

- Use the vice to clamp the work and properly support the over-hanging portion or the workpiece level with the base of the machine.

- Do not use a length stop on the free off-cut end of a clamped workpiece.

- Allow the machine to reach full speed before contacting the workpiece.

- Do not have any part of your body in line with the path of the abrasive disc.

- Ease the abrasive disc against the work piece when starting to cut. Do not force tool.

- Keep hands away from cutting area. Do not attempt to remove cut material while wheel is moving.

- After finishing the cut, release the switch, hold the saw arm down and wait for the disc to stop before removing work or off-cut piece.

- Disconnect the plug from the power source and bring the machine to a complete standstill before making any adjustments.

HOUSEKEEPING

- Ensure ventilation openings and switch levers are kept clean and free of foreign matter.

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

	Metal sparks								Noise				Sharp metal burrs

	Contact with rotating disc					Eye injuries			Burns from workpiece

FORBIDDEN

	Do not grind on the side of abrasive cut off wheels