# SAFETY OPERATING PROCEDURES

Compound Mitre Saw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure the saw is properly secured to a worktable by bolts/ clamps at approximately hip height.

- Ensure the saw is operated on an RCD protected circuit.

- Check workspaces and walkways to ensure no slip/trip-hazards are present.

- Check that all safety guards are in position and are operational.

- Ensure you are familiar with the operation of the ON/OFF starter.

- Keep table and work area clear of all tools, off-cut timber and sawdust.

- Start the dust extraction unit before using the machine.

- Faulty equipment must not be used. Immediately report suspect equipment.

OPERATIONAL SAFETY CHECKS

Do not use this saw to cut pieces that are too small to be securely clamped.

- The maximum cut for the machine must not be exceeded.

- Ensure all adjustments are secure before making a cut.

- Use clamps to secure and support the workpiece to a stable platform. Do not use a length stop on the free scrap end of a clamped workpiece.

- Do not cut more than one workpiece at a time.

- Before turning on the saw, perform a dry run of the cutting operation to ensure that no problems will occur when the cut is made.

- Do not start the saw with the blade touching the workpiece. Allow the blade to reach full speed before contacting the workpiece.

- Avoid reaching over the saw line. Do not cross arms when cutting.

- When using the right hand to pull the saw down, keep the left hand, especially the thumb well clear of the line of cut.

- If workpiece is bowed or warped, clamp it with the outside bowed face toward the fence.

- After finishing the cut, release the switch, hold the saw arm down and wait for blade to stop before removing work or off-cut piece.

- Disconnect the plug from the power source and bring the machine to a complete standstill before making any adjustments.

HOUSEKEEPING

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

- Saw may grab and ‘kick-back’ toward operator 		Flying chips and airborne dust

	Contact with rotating blade							Eye injuries						Noise

FORBIDDEN

- Cutting branches, wood with embedded nails or screws			Cutting dowel

- Ripping solid timber along the grain							Cutting ferrous or non-ferrous material