# SAFETY OPERATING PROCEDURES

Belt Disc Sander

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Gloves must not be worn when using this machine. |

### Only one person may operate this machine at any one time.

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip hazards are present.

- Ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Check the table is set not more than 2mm from disc.

- Check belts and discs are in a serviceable condition.

- Operator must be positioned out of direct line of abrasive belt at all times.

- Ensure dust extraction is on before operating machine.

- Faulty equipment must not be used. Immediately report suspect machinery.

OPERATIONAL SAFETY CHECKS

- Allow machine to reach maximum revolutions before operating to avoid overloading.

- Always place material on the table on the downward side of the disc travel to hold it down on the table surface.

- Hold material firmly against stops or table before applying pressure on abrasive.

- Keep fingers clear of disc or belt while sanding.

- Never leave the machine while it is running.

- Before making adjustments switch off and bring the machine to a complete standstill.

HOUSEKEEPING

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

#### 	Abrasions				Burns to skin			Eye injuries

	Excessive dust 		Unsecured material being flung around

FORBIDDEN:

- Attempting to sand very small items

- Attempting to sharpen tools

- Attempting to sand metal