# SAFETY OPERATING PROCEDURES

Surface Planer

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces are clear and no slip/trip-hazards are present.

- Check safety guards are adjusted and operate to give maximum protection.

- Where a bridge guard is fitted adjust the guard to ensure a maximum of 2mm clearance between guard and the timber.

- Do not plane stock with structural defects.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Set depth of cut and lock table. Never make a single cut greater than 2mm.

- Check and lock fence in position.

- Start the dust extraction unit before using the machine.

- Faulty equipment must not be used. Immediately report suspect equipment.

## OPERATIONAL SAFETY CHECKS

- Hands must not be closer than 100mm from the cutter head when it is rotating.

- Use push blocks wherever possible.

- Never leave the machine while it is running.

- Place cupped boards with the concave side against the table.

- Plane with the grain. Hold the workpiece firmly and apply even feed rate.

- Operator should stand to side of infeed table to avoid possible kickbacks.

- Before making any adjustments switch off and wait for the cutter head to completely stop.

###### HOUSEKEEPING

- Switch off and reset all guards to a fully closed position after use.

- Reset the depth of cut to zero after use.

- Leave the machine in a safe, clean and tidy state.

FORBIDDEN

- Do not surface stock less than 300mm long x 20mm wide x 15mm thick

- Rebating at the end of the cutter-block

	Planing end grain