# SAFETY OPERATING PROCEDURES

Router Table

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip hazards are present.

- Locate and ensure you are familiar with the operation of the ON/OFF starter and E-Stop (if fitted).

- Use only materials free from defects.

- Ensure cutter size conforms to machine specification.

- Machine must be isolated while cutters are being installed.

- Ensure all cutters are sharp and free of resin build up.

- Check cutter for clearance of rotation before starting machine.

- Adjust fence, guards and extraction for maximum protection and efficiency.

- Do not store tools and parts on top of the machine.

- Faulty equipment must not be used. Immediately report suspect machinery.

#### OPERATIONAL SAFETY CHECKS

- Never leave the machine running unattended.

- Before making adjustments switch off and bring the machine to a complete standstill.

- Feeding must be against the direction of rotation.

- Deep cuts should not be attempted.

- Feed work slowly into the cutter.

- Jigs, fixtures and templates should be used wherever possible.

#### HOUSEKEEPING

- Switch off the machine.

- Leave the machine in a safe, clean and tidy state.

## POTENTIAL HAZARDS

- Material kickback

	Flying chips and airborne dust

- Noise

	Eye injuries

- Contact with cutter at point of operation and during cleaning and adjustment