# SAFETY OPERATING PROCEDURES

Radial Arm Saw

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Hearing protection must be worn when using this machine. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure no slip/trip-hazards are present.

- Check that all safety guards are in position and are operational.

- Ensure you are familiar with the operation of the ON/OFF starter and E-Stop.

- Keep table and work area clear of all tools, off-cut timber and sawdust.

- Start the dust extraction unit before using the machine.

- Faulty equipment must not be used. Immediately report suspect equipment.

OPERATIONAL SAFETY CHECKS

Do not hold material in ‘hand hazard zone’. Keep hands well clear of line of cut.

- The workpiece must be held against a fence.

- Allow the saw blade to obtain maximum speed before making a cut.

- Operate the saw with the left hand where possible.

- Avoid reaching over the saw line. Do not cross arms when cutting.

- When using the right hand to pull the saw across, keep the left hand, especially the thumb well clear of the line of cut.

- Return the cutting head to the rear of the table after each cross cut.

- The maximum cut for the machine must not be exceeded.

- When cutting bowed timber place the bow against the table to avoid the saw binding.

- Before making adjustments switch off and bring the machine to a complete standstill.

HOUSEKEEPING

- Switch off the saw and reset all guards to a fully closed position.

- Leave the machine in a safe, clean and tidy state.

POTENTIAL HAZARDS

- Saw may grab and ‘kick-back’ toward operator 		Flying chips and airborne dust

	Contact with rotating blade		Eye injuries		Noise

FORBIDDEN

- Cutting branches, wood with embedded nails or screws		Cutting dowel

- Ripping solid timber along the grain							Cutting short lengths of timber