# SAFETY OPERATING PROCEDURES

Manual Metal Arc Welder

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in addition to welding mask. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing to cover arms and legs must be worn. |
| Respiratory protection devices may be required for some operations. | Oil free leather gloves and spats must be worn when welding. |
| Rings and jewellery must not be worn. | A welding mask with correct grade lens for MMAW must be worn. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Ensure the work area is clean and clear of grease, oil, and any flammable materials.

- Keep the welding equipment and work area dry to avoid electric shocks.

- Ensure the gloves are dry and free from holes to avoid electric shocks.

- Ensure electrode holder and work leads are in good condition.

- Ensure fume extraction unit is on before beginning welding operation.

- Ensure other people are protected from flashes by closing curtain to welding bay or by erecting screens.

- Faulty equipment must not be used. Immediately report suspect equipment.

#### OPERATIONAL SAFETY CHECKS

- Keep welding leads as short as possible and coil them to minimise inductance.

- Ensure the electrode holder has no electrode in it before turning on the welding machine.

- Ensure current is correctly set according to electrode selection.

- Do not use bare hands and never wrap electrode leads around yourself.

- When welding is finished or interrupted, remove electrode stub from holder and switch off power source.

#### HOUSEKEEPING

- Switch off the machine and fume extraction.

- Hang up electrode holder and welding cables and leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Electric shock		Fume			Radiation burns to eyes or body

	Body burns due to hot or molten materials			Flying sparks			Fire