# SAFETY OPERATING PROCEDURES

Oxy-Fuel Gas Welding

DO NOT use this equipment unless a teacher has instructed

you in its safe use and operation and has given permission.

| Welding goggles must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Oil free leather gloves must be worn when welding. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Keep area clean and free of grease, oil and other flammable material.

- Gas hoses must be in good condition and not create a tripping hazard.

- Before lighting up make a visual inspection of all equipment for damage.

- Check that the area is well ventilated and fume extraction unit is on.

- Faulty equipment must not be used. Immediately report suspect equipment.

##### PRESSURE SETTING

- Check that the oxygen and acetylene regulator adjusting knobs are loose.

- Check that both blowpipe valves are closed.

- Slowly open the cylinder valves on each cylinder for half a turn only.

- Screw in the regulator adjusting knobs slowly until the delivery pressure gauges register 70kPa.

- Purge and check for constant oxygen gas flow. 
	Open the oxygen blowpipe for 2 seconds valve and check the delivery gauge.
	If necessary re-adjust the oxygen regulator to achieve a 70kPa pressure. 
	Close the oxygen blowpipe valve.

- Purge and check for constant acetylene gas flow. 
	Open the acetylene blowpipe valve for 2 seconds and check the delivery gauge.
	If necessary re-adjust the acetylene regulator to achieve a 70kPa pressure. 
	Close the acetylene blowpipe valve.

LIGHTING UP

- Open the acetylene blowpipe valve slightly and light the blowpipe with a flint lighter.

- Continue to slowly open the acetylene valve until the flame no longer produces soot.

- Slowly open the oxygen blowpipe valve until a neutral flame is produced.

###### SHUTTING OFF BLOWPIPE

- Close the acetylene blowpipe valve first.

- Then close the oxygen blowpipe valve.

##### CLOSING DOWN

- Close down both cylinder valves.

- Open oxygen blowpipe valve to allow the gas to drain out.

- When oxygen gauges read zero, unscrew regulator-adjusting knob.

- Close oxygen blowpipe valve.

- Turn off acetylene cylinder valve.

- Open acetylene blowpipe valve and release gas.

- When acetylene gauges read zero, release regulator adjusting knob.

- Close acetylene blowpipe valve.

#### HOUSEKEEPING

- Hang up welding blowpipe and hoses.

- Turn off fume extraction.

- Leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

## 	Burns									Flying sparks 						Fumes

## 	Oil and grease   					Combustible materials			Flashbacks

	Radiation damage to eyes		Explosion by gas leakage

FORBIDDEN

##### 	Lighting blowpipe with matches or lighters

    - Using oil, grease or other hydrocarbons

    - Using oxygen as a substitute for compressed air