# SAFETY OPERATING PROCEDURES

Spot Welder

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in work areas. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing must be worn. |
| Rings and jewellery must not be worn. | Leather gloves must be worn when handling hot metal. |

### PRE-OPERATIONAL SAFETY CHECKS

- Check workspaces and walkways to ensure that no slip/trip hazards are present.

- Check switchgear and cable are in sound condition.

- Check electrode points are in good condition and meet exactly.

- Ensure electrodes are securely mounted and clean from contaminants.

- Set pressure on clamps to hold work securely without damaging work.

- Preset weld time under teacher direction (if timer fitted).

- Gloves should be used to position and hold work.

- Faulty equipment must not be used. Immediately report suspect machinery.

OPERATIONAL SAFETY CHECKS

- Ensure the spot welder has cooled before making any adjustments.

- Avoid prolonged use due to heat build-up of electrodes and arms.

- Take care when holding work due to heat created during welding process.

HOUSEKEEPING

- Switch off the spot welder.

- Leave the work area in a safe, clean and tidy state.

#### POTENTIAL HAZARDS

- Electrodes become hot with continued use

	Burns

- Hot metal

	Spitting metal or flying sparks

	Eye injuries