# SAFETY OPERATING PROCEDURES

Gas Tungsten Arc Welder

DO NOT use this machine unless a teacher has instructed

you in its safe use and operation and has given permission.

| Safety glasses must be worn at all times in addition to welding mask. | Long and loose hair must be contained. |
| --- | --- |
| Appropriate footwear with substantial uppers must be worn. | Close fitting/protective clothing to cover arms and legs must be worn. |
| Respiratory protection devices may be required for some operations. | Oil free leather gloves and spats must be worn when welding. |
| Rings and jewellery must not be worn. | A welding mask with correct grade lens for GTAW must be worn. |

### PRE-OPERATIONAL SAFETY CHECKS

- Ensure no slip/trip hazards are present in workspaces and walkways.

- Ensure the work area is clean and clear of grease, oil, and any flammable materials.

- Keep the welding equipment, work area and gloves dry to avoid electric shocks.

- Ensure gloves, welding torch and work leads are in good condition.

- Ensure other people are protected from flashes by closing curtain to welding bay or erecting screens.

- Ensure fume extraction unit is on before beginning welding operation.

- Ensure work leads do not create a tripping hazard.

- Faulty equipment must not be used. Immediately report suspect equipment

#### OPERATIONAL SAFETY CHECKS

- Ensure machine is correctly set up for current, voltage, and gas flow.

- Ensure work return cables make firm contact to provide a good electrical connection.

- Strike the arc before placing the tip of the filler wire in the weld zone.

- Turn of the power while changing tungsten electrodes.

- Never leave the welder running unattended.

- Take care to avoid flashes.

#### HOUSEKEEPING

- Switch off the machine and fume extraction.

- Close gas cylinder valve.

- Hang up welding gun and leads.

- Leave the work area in a safe, clean and tidy state.

## POTENTIAL HAZARDS

	Electric shock		Fume			Radiation burns to eyes or body

	Body burns due to hot or molten materials			Flying sparks			Fire