SCAFFOLDS – METAL FRAME

Contract Name and Number:				Contractor/Subcontractor:

Government Inspector:				Location:

Contractor Inspector:					Date:

Listed below are the minimum basic requirements for metal frame scaffolds, taken from                  YES     NO      N/A

EM 385-1-1

1.  Does the scaffold have a standard guardrail system including toeboards?  (21.B)

2.  Are scaffolds and their components capable of supporting without failure at least

4 times the maximum anticipated load?  (22.B.01a)

3.  Are scaffolds plumb and level?  (22.B.04)

4.  Do scaffolds (other than suspended scaffolds) bear on base plates upon sills or

other adequate foundation?  (22.B.04)

5.  Are working levels fully planked or decked? (22.B.05)

6.  Is all planking overlapped or secured from movement?  (22.B.06a)

7.  Do scaffold planks extend over their end supports not less than 6 inches (unless planking is

manufactured with restraining hooks or equivalent means) nor more than 12 inches?  (22.B.06b)

8.  Is planking supported or braced to prevent excessive spring or deflection and secured and

supported to prevent loosening, tipping, or displacement?  (22.B.06e)

9.  Has a ladder or equivalent safe access been provided?  (22.B.08a)

10.  If a built-in ladder is part of a scaffold system, does it conform to the requirements for

ladders?  (22.B.08b)

11.  When the scaffold height exceeds 4 times the minimum scaffold base dimension, is the

scaffold secured to the wall or to the structure?  (22.B.09)

12.  Is the use of brackets on scaffolds prohibited unless the tipping effect is controlled? (22.B.10)

13.  Are sections of metal scaffolds securely connected and all braces securely fastened?(22.C.02)

14.  Do all tube and coupler scaffolds have posts, runners, bracing of nominal 2-inch steel tubing

or pipe?  (22.C.04a)

15.  Are all tube and coupler scaffolds constructed to support 4 times the maximum intended

loads?  (22.C.04a)

16.  Are metal frame scaffolds properly braced by cross, horizontal, or diagonal braces (or

combination of these) to secure vertical members together laterally?  (22.C.05b)

17.  Do all wheels and casters on rolling scaffolds have a positive locking device, securely

fastened to the scaffold, to prevent accidental movement? (22.C.06a)

18.  Is the height of a free-standing mobile scaffold no more than three times the smallest base

dimension?  (22.C.06e)

Comments:

This checklist is based on EM 385-1-1, dated 3 September 1996.  Use of this checklist is optional.

SCAFFOLDS – OTHER TYPES

Contract Name and Number:				Contractor/Subcontractor:

Government Inspector:				Location:

Contractor Inspector:					Date:

Listed below are the minimum basic requirements for metal frame scaffolds, taken from                  YES     NO      N/A

EM 385-1-1

1.  Does the scaffold have a standard guardrail system including toeboards?  (21.B)

2.  Are scaffolds and their components capable of supporting without failure at least

4 times the maximum anticipated load?  (22.B.01a)

3.  Are scaffolds plumb and level?  (22.B.04)

4.  Do scaffolds (other than suspended scaffolds) bear on base plates upon sills or

other adequate foundation?  (22.B.04)

5.  Are working levels fully planked or decked? (22.B.05)

6.  Is all planking overlapped or secured from movement?  (22.B.06a)

7.  Do scaffold planks extend over their end supports not less than 6 inches (unless planking is

manufactured with restraining hooks or equivalent means) nor more than 12 inches?  (22.B.06b)

8.  Is planking supported or braced to prevent excessive spring or deflection and secured and

supported to prevent loosening, tipping, or displacement?  (22.B.06e)

9.  Has a ladder or equivalent safe access been provided?  (22.B.08a)

10.  If a built-in ladder is part of a scaffold system, does it conform to the requirements for

ladders?  (22.B.08b)

11.  When the scaffold height exceeds 4 times the minimum scaffold base dimension, is the

scaffold secured to the wall or to the structure?  (22.B.09)

12.  Is the use of brackets on scaffolds prohibited unless the tipping effect is controlled? (22.B.10)

13.  Are  all wood scaffolds 60 feet or less in height constructed in accordance with Table 22-2?

(22.D.01)

14.  Where wood poles are spliced, are the ends squared and the upper section rests squarely

on the lower section?  (22.D.03a)

15.  Are ledgers long enough to extend over a minimum of two poles and reinforced by bearing

blocks nailed to the side of the pole to form a support for the ledger?  (22.D.04c)

16.  Are independent pole scaffolds set as near to the wall of the building as possible?  (22.C.05b)

17. Have the suspended scaffold, hoist, ropes, brakes, and anchorages been inspected prior to

being placed in service? (22.E.02)

18.  Do all parts of suspended scaffolds have a minimum safety factor of 4 and support ropes a

minimum safety factor of 6?  (22.E.04)

19.  Are outrigger beams made of structural metal and restrained to prevent movement?(22.E.07a)

20.  Does each hoist have a name plate with the manufacturer’s name, maximum load rating,

identification number, and wire rope specifications?  (22.E.08b)

This checklist is based on EM 385-1-1, dated 3 September 1996.  Use of this checklist is optional.

SCAFFOLDS – OTHER TYPES (CONT)                         YES     NO     N/A

21.  Are suspended scaffolds guyed, braced, guided, or equipped with a tag line to prevent

swaying?  (22.E.10)

22.  Are two-point suspension scaffold platforms between 20 and 36 inches wide and securely

fastened to the hangers?  (22.E.11a)

23.  Is each person supported by a suspended scaffold protected by a Type I body belt or body

harness system attached by lanyard to a lifeline, trolley line, or scaffold structural member?

(22.E.15)

24.  Is each bracket, except for wooden bracket form scaffolds, attached to the supporting

formwork or structure by nails, metal stud attachment device, welding, hooking or a secured

structure supporting member, or, for a carpenter’s bracket scaffold only, by a bolt through

the opposite side of the structure’s wall?  (22.G.02)

25.  Are horse scaffolds constructed or arranged in two tiers or less?  (22.H.01)

26.  Do pump jack scaffolds carry a working load of 500 pounds or less?  (22.I.01)

Comments:

This checklist is based on EM 385-1-1, dated 3 September 1996.  Use of this checklist is optional.